Activity or Experiment-Based Risk Assessment - Activity Sequence List

College of Design & Engineering

| Experiment / Activity Steps |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Department: |  | Mechanical Engineering |  | Location: | E2-01-04 | Laboratory: | Control & Mechatronics |
| No. | Experiment / Activity |  | Workable Sequence/Task |  |  |  |  |
| 1 | Franka Research 3 Robot Operation |  | System setup, cable check and power-on |  |  |  |  |
| 2 | Franka Research 3 Robot Operation |  | Programming, teaching and calibration |  |  |  |  |
| 3 | Franka Research 3 Robot Operation |  | Running robot motion / trajectory testing |  |  |  |  |
| 4 | Franka Research 3 Robot Operation |  | End-effector and payload handling |  |  |  |  |
| 5 | Franka Research 3 Robot Operation |  | Shutdown, recovery and housekeeping |  |  |  |  |
| 6 | Aloha Robot Experiment |  | System setup and power-on |  |  |  |  |
| 7 | Aloha Robot Experiment |  | Using Aloha Teleoperation System to collect data |  |  |  |  |
| 8 | Aloha Robot Experiment |  | Training model on workstation/computer |  |  |  |  |
| 9 | Aloha Robot Experiment |  | Deployment and testing of trained model |  |  |  |  |
| 10 | Aloha Robot Experiment |  | Shutdown and housekeeping |  |  |  |  |
| 11 | Dexterous Hand Hardware Setup |  | 3D printed parts post-processing (support removal, sanding, filing), visual inspection of printed finger joints and palm structure |  |  |  |  |
| 12 | Dexterous Hand Hardware Setup |  | Mechanical assembly (fitting motors, gears, tendon strings/cables into finger joints), mounting finger linkages onto palm base, gear meshing and joint alignment |  |  |  |  |
| 13 | Dexterous Hand Hardware Setup |  | Electrical wiring & PCB soldering (motor drivers, position/force sensors, communication cables), connecting power and data lines to controller board |  |  |  |  |
| 14 | Dexterous Hand Hardware Setup |  | System integration (mounting hand onto test stand, connecting power supply & communication cables), cable routing with strain relief, securing all connectors |  |  |  |  |
| 15 | Dexterous Hand Hardware Setup |  | Initial power-on & firmware flashing/calibration (finger homing, zero-position setting, joint angle initialization) |  |  |  |  |
| 16 | Dexterous Hand Hardware Setup |  | Functional testing (open/close grip, individual finger tracking, payload lifting with lightweight objects), trajectory testing and force control validation |  |  |  |  |

| Activity or Experiment-Based Risk Assessment Form |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Laboratory: | Control & Mechatronics | Name of Experiment/Activity: | System design, testing, and data collecting |  | FYP/Project/Activities |
| Location: | E2-01-04 | Name of student/RF/RE/RS/staff: | <APPLICANT_NAME> | Name of PI/Supervisor: | <SUPERVISOR_NAME> |
| Last Review Date: | <DATE> | Next Review Date: | <DATE> |  |  |

| 1. Hazard Identification |  |  |  | 2. Risk Evaluation & Control |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| No. | Task | Hazards | Possible Consequences | Existing Risk Control (if any) | S | L | R | Additional / New Risk Control | S | L | R | Action By | Deadline |
| 1 | System setup and power-on | Electrical shock from cables/plugs; trip hazard from loose cables | Electric shock, fall, bruises | Visual inspection before use Use proper power supply Keep cables organized | 2 | 2 | 4 | Secure/tape cables Do not use damaged wires Power off before adjustment | 2 | 1 | 2 | <APPLICANT_NAME> | Before each use |
| 2 | Teleoperation data collection | Moving robot arms/grippers (pinch/collision) | Finger injury, bruises, equipment damage | Only trained user operates Keep clear of robot workspace Emergency stop accessible Start with low speed | 2 | 2 | 4 | Define safety zone Do not reach into workspace during motion Stop system before adjustment | 2 | 1 | 2 | <APPLICANT_NAME> | Before operation |
| 3 | Deployment/testing | Unexpected robot motion, dropped objects | Impact injury, equipment damage | Clear workspace Low speed testing Emergency stop ready | 2 | 2 | 4 | Supervisor present for initial tests Validate program before running Only one operator | 2 | 1 | 2 | <APPLICANT_NAME> | Before deployment |
| 4 | System setup and power-on | Electrical shock from power cables/controller; trip hazard from loose cables; incorrect connection or unstable mounting | Electric shock, fall, bruises, equipment damage | Visual inspection before use; use approved power supply; keep emergency stop accessible; keep cables organized | 3 | 2 | 6 | Secure/tape cables; do not use damaged wires; power off before connection or adjustment; check robot base/workbench stability before motion | 3 | 1 | 3 | <APPLICANT_NAME> | Before each use |
| 5 | Programming, teaching and calibration | Unintended motion from wrong program, frame, or trajectory; incorrect speed/joint limit setting; pinch/collision with 7-axis arm | Impact or pinch injury, bruises, damage to robot/fixture/workspace | Only trained user operates; review code/trajectory before running; use low speed for first test; emergency stop accessible | 3 | 2 | 6 | Validate in simulation or slow mode first; define clear safety zone; do not reach into workspace during motion; supervisor present for first run | 3 | 1 | 3 | <APPLICANT_NAME> | Before operation |
| 6 | Running robot motion / trajectory testing | Moving robot links and end-effector; unexpected restart/recovery motion; collision with user, table, or equipment | Crushing/pinch injury, impact injury, equipment damage | Keep clear of robot workspace; clear workspace before test; use speed/force limits where available; one operator controls the run | 3 | 2 | 6 | Announce before starting motion; use safe stop before entering workspace; wait until robot is fully stopped; no bystanders inside robot workspace | 3 | 1 | 3 | <APPLICANT_NAME> | During operation |
| 7 | End-effector and payload handling | Dropped payload/tool; loose fasteners; incorrect payload/TCP/center-of-mass setting; sharp edges on tools/parts | Hand/foot injury, cuts or bruises, damage to gripper/robot/sample | Check payload mass and mounting; use suitable screws/fixtures; support payload during installation; keep hands away from pinch points | 3 | 2 | 6 | Verify TCP, payload and center-of-mass parameters before motion; wear eye protection/gloves if needed; do not exceed payload limits; remove loose objects from workspace | 3 | 1 | 3 | <APPLICANT_NAME> | Before payload test |
| 8 | Shutdown, recovery and housekeeping | Residual robot motion; energized controller/cables; improper emergency unlocking; loose tools/cables | Pinch injury, electric shock, trip/fall, equipment damage | Stop program before shutdown; return robot to safe pose; power down controller as required; keep area tidy | 3 | 2 | 6 | Use emergency unlocking only when required and by trained personnel; support the arm before unlocking; store tools/cables after use; report faults immediately | 3 | 1 | 3 | <APPLICANT_NAME> | After each use |
| 9 | 3D Printed Parts Post-Processing (Support removal, sanding, filing) | Sharp edges on printed parts; airborne plastic/resin dust; snap of thin structural fingers during cleaning | Cuts to fingers, eye irritation, respiratory discomfort, broken printed components | Wear cut-resistant gloves; use sanding tools in well-ventilated area or fume hood; hold parts firmly near thick sections | 2 | 2 | 4 | Wear safety goggles and N95 dust mask; use a deburring tool instead of bare blades; support thin finger parts with soft foam during filing | 2 | 1 | 2 | <APPLICANT_NAME> | Before assembly |
| 10 | Mechanical Assembly (Fitting motors, gears, tendon strings/cables into finger joints) | Pinch points between joint linkages; tendon/string snap under tension; small screws/springs flying off; improper gear meshing | Finger/hand bruising, eye injury from flying parts, tendon breakage, gear skipping/damage | Use tweezers and needle-nose pliers for small parts; pre-tension tendons gradually; check gear alignment visually before tightening | 3 | 2 | 6 | Wear safety glasses during spring/tension work; use a magnetic tray to catch small fasteners; apply low-strength threadlocker on screws; use a fixture jig to hold the palm base steady | 2 | 1 | 2 | <APPLICANT_NAME> | During assembly |
| 11 | Electrical Wiring & PCB Soldering (Motor drivers, sensors, communication cables) | Burns from soldering iron; short circuit from loose solder bridges; incorrect wire polarity; ESD damage to sensitive boards | Skin burns, PCB/peripheral damage, controller failure, fire hazard | Use soldering station with tip rest; disconnect power before soldering; double-check pinout against datasheet; wear ESD wrist strap | 3 | 2 | 6 | Keep a fire-resistant mat under the board; use heat shrink tubing on exposed joints; test continuity with a multimeter before power-on; keep a small fire extinguisher nearby | 2 | 1 | 2 | <APPLICANT_NAME> | Before power-on |
| 12 | System Integration (Mounting hand onto test stand, connecting power & data cables) | Heavy hand dropping during mounting; cable strain/tension at connectors; tangled wires around moving joints | Equipment damage, connector pin breakage, intermittent signal loss, trip hazard | Use a counterbalanced arm or adjustable stand to hold the hand; route cables with strain relief loops; secure cables with spiral wrap away from joints | 2 | 2 | 4 | Have a second person assist with mounting; use locking-type aviation connectors; perform a "cable sweep test" (move all joints slowly) to check clearance before full tightening | 2 | 1 | 2 | <APPLICANT_NAME> | During integration |
| 13 | Initial Power-on & Firmware Flashing/Calibration (Finger homing, zero-position setting) | Uncontrolled "twitching" or fast homing motion; motor over-current; finger collision with test stand; unexpected reverse motion | Pinched fingers, gear train damage, motor driver burnout, hand detachment from mount | Set current/torque limits in firmware to 30% of max; start with only one finger active at a time; place soft foam blocks around the workspace | 3 | 2 | 6 | Keep the emergency stop (or software kill switch) within immediate reach; run homing at 20% speed first; observe joint angle feedback in real-time on screen; enable software position limits before enabling torque | 2 | 1 | 2 | <APPLICANT_NAME> | During first boot-up |
| 14 | Functional Testing (Open/close grip, individual finger tracking, payload lifting) | Dropped test object (e.g., weighted block) from gripper; fingertip collision with table/surroundings; over-torque during sustained holding | Foot/equipment impact damage, fingertip wear, motor overheating, object projectile | Test with lightweight (≤50g) soft objects initially; gradually increase payload; define a virtual "safety cylinder" workspace in software; monitor motor temperature via thermal camera | 3 | 2 | 6 | Install a transparent acrylic shield around the test area; announce "motion starting" before each run; set a 1-second delay before high-torque engagement; perform a "dry run" in simulation mode first (if available) | 2 | 1 | 2 | <APPLICANT_NAME> | Before each test run |

| Conducted by: (Name, designation) | <APPLICANT_NAME>, Ph.D. student |  | Approved by: (Name, designation) |  | <SUPERVISOR_NAME>, assistant professor |  |
| --- | --- | --- | --- | --- | --- | --- |
| Signature: |  |  | Signature: |  |  |  |
| Date: | <DATE> |  | Date: |  | <DATE> |  |
| Conducted in the presence of: (name of student/RF/RE/RS/staff) |  |  |  |  |  |  |
| Signature: |  |  |  |  |  |  |
| Date: |  |  |  |  |  |  |

| Likelihood |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Almost Certain | Likely | Possible | Unlikely | Rare |
| Severity | Insignificant | 5 | 4 | 3 | 2 | 1 |
|  | Minor | 10 | 8 | 6 | 4 | 2 |
|  | Moderate | 15 | 12 | 9 | 6 | 3 |
|  | Major | 20 | 16 | 12 | 8 | 4 |
|  | Extreme | 25 | 20 | 15 | 10 | 5 |

Risk = Likelihood x Severity

Risk Assessment Matrix

| RISK | DECISION PROCESS |
| --- | --- |
| < 5 | RISK ACCEPTABLE |
| 5 – 14 | CONSIDER ADDITIONAL RISK CONTROL |
| > 14 | ADDITIONAL RISK CONTROL REQUIRED |

|  | Likelihood |  |
| --- | --- | --- |
| 1 | Rare | Very unlikely to occur |
| 2 | Unlikely | Not likely to occur |
| 3 | Possible | May occur occasionally |
| 4 | Likely | Common or repeating occurrence |
| 5 | Almost Certain | Expected to occur |
|  | Severity |  |
| 1 | Insignificant | Negligible injury or ill-health, including mental well-being |
| 2 | Minor | Injury or ill-health, including mental well-being, requiring first-aid only; includes minor cuts and bruises, irritation, ill-health with temporary discomfort, fatigue, temporary/minor psychological stress |
| 3 | Moderate | Injury or ill-health, including mental well-being, requiring medical treatment; includes lacerations, burns, sprains, minor fractures, psychosocial stress requiring counselling, dermatitis, and work-related musculoskeletal disorders |
| 4 | Major | Major injuries, serious occupational diseases or exposure; includes amputations, major fractures, multiple injuries, occupational cancer, diagnosed mental illnesses requiring medical intervention, acute poisoning, disabilities, and noise-induced hearing loss |
| 5 | Extreme | Death, fatal occupational diseases or exposure, or multiple major injuries |
|  | Severity - Consider the magnitude/severity of the consequences of the Risk Factor occurring and then list this as 5 (Extreme), 4 (Major), 3 (Moderate), 2 (Minor), or 1 (Insignificant). Severity normally will not change unless there is a physical change to the equipment or process. |  |
|  | Likelihood - Team should rely upon their experience and consider realistic scenarios. Listed below are examples of factors that may be considered in determining the likelihood. - Past experience / incidents - Complexity of the activity - Number of personnel involved in the activity (e.g. all personnel, a limited number of trained personnel, etc) - Frequency of use or execution - Degree of control (involvement of contractors) - Strength/completeness of administrative controls - Sufficiency/formality of training - Other.... |  |